# 131   CODED DATA FIELD:  CARTOGRAPHIC MATERIALS:  GEODETIC, GRID AND VERTICAL MEASUREMENT

## Field Definition

This field contains coded data relating to the geodetic, grid and vertical measurement details of cartographic materials.

## Occurrence

Optional.  Not repeatable.

## Indicators

Indicator 1:  blank (not defined)

Indicator 2:  blank (not defined)

## Subfields

$a	Spheroid

A two-character code indicates the spheroid used to construct the map.  Repeatable.

$b	Horizontal Datum

A three character code used to indicate the horizontal datum(s) named on the cartographic material.  Repeatable for each datum.

$c	Grid and Referencing System

A two-character code indicating the main grid or referencing system.  This code (and those in subfields $d and $e) includes the spheroid of construction of the grid (as opposed to the spheroid of construction of the map given in subfield $a). Repeatable.

$d	Overlapping and Referencing System

A two-character code indicating a subsidiary grid or referencing system which extends across the face of the cartographic material.  Repeatable.

$e	Secondary Grid and Referencing System

A two-character code indicating a subsidiary grid or referencing system which appears in the form of marginal ticks.  Repeatable.

$f	Vertical Datum

A two-character code used to indicate the vertical datum(s) named on the cartographic material.  Repeatable for each datum.

$g	Unit of Measurement of Heighting

A two-character code indicating the unit of measurement of heighting.  Repeatable.

$h	Contour Interval

Up to four characters indicating the value of interval of the main contours (i.e. those contours which always appear when applicable) in the unit of measurement given in subfield $g. Up to one decimal place may be recorded, values of more than one decimal place should be rounded up to one. Repeatable for each value when, for example, the value changes with height.

$i	Supplementary Contour Interval

Up to four characters indicating the value of the interval of supplementary contours, i.e. contours used between main contours to increase the topographic expression of the area (e.g. in areas of low relief) or contours whose value is not repeated on the surface of the map.  The unit of measurement is that given in subfield $g. Up to one decimal place may be recorded, values of more than one decimal place should be rounded up to one. Repeatable.

$j	Unit of Measurement of Bathymetry

$k	Bathymetric Interval

$l	Supplementary Bathymetric Interval

The oceanographic equivalent of subfields $g to $i. The rules for these subfields apply to subfields $j to $1.

## Notes on Field Contents

For a list of codes see Appendix F.

Related Fields

120	CODED DATA FIELD:  CARTOGRAPHIC MATERIALS – GENERAL

121	CODED DATA FIELD: CARTOGRAPHIC MATERIALS – 	
PHYSICAL ATTRIBUTES

These fields are used to code other attributes of cartographic materials.

Examples

EX 1:	131 ##$auu$baqh$cpe

The spheroid used to construct the map is unknown.  The horizontal datum is the European Datum.  The map uses the Universal Transverse Mercator Grid, International Spheroid.

EX 2:	13l ##$auu$bbpm$cpd$fnk$gab$h40$i20

The spheroid used to construct the map is unknown.  The horizontal datum is the North American Datum 1927. The map uses the Universal Transverse Mercator Grid, unknown spheroid. The vertical datum is the Sea Level Datum 1929, USA. The heighting is in feet.  Main contour intervals are at 40 feet, with other intervals at 20 feet.